# Reference Guide: Cisco and Arista IPsec VTI Tunnel with BGP and DDNS

## Cisco IOS XE Configuration

IPsec VTI Tunnel Configuration:

crypto isakmp policy 10
 encr aes 256
 hash sha256
 authentication pre-share
 group 14

crypto isakmp key <tenant-specific-psk> address <strongswan-public-ip>

crypto ipsec transform-set TS esp-aes 256 esp-sha-hmac

crypto ipsec profile IPSEC-VTI
 set transform-set TS
 set isakmp-profile default

interface Tunnel100
 ip address 10.10.100.2 255.255.255.252
 tunnel source Loopback0
 tunnel destination <strongswan-ddns-hostname>
 tunnel mode ipsec ipv4
 tunnel protection ipsec profile IPSEC-VTI

## Arista EOS Configuration

IPsec VTI Tunnel Configuration:

interface Tunnel100
 ip address 10.10.100.2/30
 tunnel source Loopback0
 tunnel destination <strongswan-ddns-hostname>
 tunnel mode ipsec ipv4
 tunnel protection ipsec profile IPSEC-VTI

ip access-list IPSEC-ACL
 10 permit ip any any

crypto isakmp policy 10
 encryption aes 256
 hash sha256
 authentication pre-share
 group 14

crypto isakmp key <tenant-specific-psk> address <strongswan-public-ip>

crypto ipsec transform-set TS esp-aes 256 esp-sha-hmac

crypto ipsec profile IPSEC-VTI
 set transform-set TS
 set isakmp-profile default

## BGP Configuration

router bgp 65010
 bgp router-id 10.10.100.2
 neighbor 10.10.100.1 remote-as 65001
 address-family ipv4
  network 10.10.100.0 mask 255.255.255.252
  neighbor 10.10.100.1 activate
 exit-address-family

## DDNS Integration

The strongSwan node uses ddclient to update a public DNS record (e.g., vpn.tenant-a.example.com) with its loopback IP. Cisco and Arista devices use this hostname as the tunnel destination, allowing dynamic tunnel anchoring.

Sample ddclient.conf:

protocol=dyndns2
use=web
server=dyndns.example.com
login=yourlogin
password=yourpassword
203.0.113.1

## IP Anycast Functionality

IP Anycast allows multiple nodes to share the same IP address for tunnel termination. Each node advertises the same loopback IP (e.g., 203.0.113.1/32) via BGP. Routing protocols direct traffic to the nearest or most available node, enabling failover and load distribution without changing DNS records.

## Tunnel Failover Test Plan

1. Ensure all nodes advertise 203.0.113.1/32 via BGP.

2. Initiate IPsec tunnel to vpn.example.com and verify endpoint.

3. Simulate node failure and observe tunnel re-establishment.

4. Validate DDNS updates and DNS resolution.

5. Confirm routing and traffic path via traceroute or tcpdump.

## Automation Script Using Expect

This script automates configuration delivery to Cisco and Arista devices using expect:

#!/usr/bin/expect -f
set timeout 20
set device_ip [lindex $argv 0]
set username [lindex $argv 1]
set password [lindex $argv 2]
set config_file [lindex $argv 3]

spawn ssh $username@$device_ip
expect "Password:"
send "$password\r"
expect ">"
send "enable\r"
expect "#"
send "configure terminal\r"
expect "(config)#"

set fp [open $config_file r]
set config_data [read $fp]
close $fp

foreach line [split $config_data "\n"] {
    send "$line\r"
    expect "(config)#"
}

send "end\r"
expect "#"
send "write memory\r"
expect "#"
send "exit\r"