# Extended Deployment Guide: Multi-Tenant Architecture on Rocky Linux

## IP Anycast Functionality

IP Anycast allows multiple nodes to share the same IP address for tunnel termination. Each node advertises the same loopback IP (e.g., 203.0.113.1/32) via BGP. Routing protocols direct traffic to the nearest or most available node, enabling failover and load distribution without changing DNS records.

## Anycast BGP Configuration

Sample BGP configuration using FRR:

router bgp 65001
 bgp router-id 10.10.100.1
 neighbor 10.10.100.2 remote-as 65002
 neighbor 10.10.100.3 remote-as 65003
 address-family ipv4 unicast
  network 203.0.113.1/32
 exit-address-family

## DDNS Integration

DDNS ensures that the DNS record (e.g., vpn.example.com) always resolves to the current public IP of the loopback interface. Tools like ddclient monitor IP changes and update DNS records automatically, enabling dynamic tunnel binding.

Sample ddclient.conf:

protocol=dyndns2
use=web
server=dyndns.example.com
login=yourlogin
password=yourpassword
203.0.113.1

## Tunnel Failover Test Plan

1. Ensure all nodes advertise 203.0.113.1/32 via BGP.

2. Initiate IPsec tunnel to vpn.example.com and verify endpoint.

3. Simulate node failure and observe tunnel re-establishment.

4. Validate DDNS updates and DNS resolution.

5. Confirm routing and traffic path via traceroute or tcpdump.

## IPsec VTI Configuration with Pre-Shared Keys

Example /etc/ipsec.conf:

conn tenant-a
    left=%any
    leftid=@node1.example.com
    leftsubnet=0.0.0.0/0
    right=%any
    rightid=@node2.example.com
    rightsubnet=0.0.0.0/0
    auto=start
    keyexchange=ikev2
    ike=aes256-sha256-modp2048
    esp=aes256-sha256
    authby=secret
    mark=100
    vti-interface=vti100
    vti-routing=yes

Add the following to /etc/ipsec.secrets:

@node1.example.com @node2.example.com : PSK "<tenant-specific-key>"

## Pre-Shared Key Generation and Delivery Script

This script generates a unique PSK for each tenant and sends it via email using sendmail:

#!/bin/bash

TENANTS=("tenant-a" "tenant-b" "tenant-c")
EMAILS=("admin-a@example.com" "admin-b@example.com" "admin-c@example.com")

for i in ${!TENANTS[@]}; do
  TENANT=${TENANTS[$i]}
  EMAIL=${EMAILS[$i]}
  PSK=$(openssl rand -hex 32)
  echo "@node1.example.com @${TENANT}.example.com : PSK "$PSK"" > /etc/ipsec.d/${TENANT}.secrets

  echo "Sending PSK for $TENANT to $EMAIL"
  echo -e "Subject: PSK for $TENANT\n\nYour pre-shared key is: $PSK" | sendmail $EMAIL
done