# Deployment Guide: Multi-Tenant Architecture on Rocky Linux (3-Node)

This guide provides step-by-step instructions to deploy a multi-tenant network architecture on three Rocky Linux nodes. The solution includes IPsec tunnels using VTI interfaces, VRFs for routing isolation, VXLAN overlays for cross-node extension, BGP with private ASNs, per-VRF traffic filtering, outbound NAT with unique IPs, and tunnel termination via internet-routable loopbacks using DDNS and IP anycast.

## 1. Base System Preparation

Each node should be installed with Rocky Linux and configured with the following components:
- strongSwan for IPsec
- iproute2 for VRF and VXLAN
- FRRouting (FRR) for BGP
- nftables for traffic filtering and NAT
- ddclient for DDNS updates
- CoreDNS or Unbound for DNS forwarding

Update system and install required packages:

sudo dnf update -y
sudo dnf install strongswan iproute iproute-tc frr nftables ddclient -y

## 2. IPsec Tunnel Setup

Configure strongSwan on each node to establish VTI-based IPsec tunnels. Use loopback IPs for tunnel termination and DDNS for dynamic updates.
Example /etc/ipsec.conf:

conn tenant-a
    left=%any
    leftid=@node1.example.com
    leftsubnet=0.0.0.0/0
    right=%any
    rightid=@node2.example.com
    rightsubnet=0.0.0.0/0
    auto=start
    keyexchange=ikev2
    ike=aes256-sha256-modp2048
    esp=aes256-sha256
    mark=100
    vti-interface=vti100
    vti-routing=yes

## 3. VRF Configuration

Create VRFs for each tenant and bind VTI interfaces to them:
ip link add vrf-tenant-a type vrf table 1001
ip link set vrf-tenant-a up
ip link set vti100 master vrf-tenant-a
ip rule add iif vrf-tenant-a table 1001
ip route add table 1001 default dev vti100

## 4. VXLAN Overlay Configuration

Extend VRFs across nodes using VXLAN:
ip link add vxlan100 type vxlan id 100 dev eth0 dstport 4789
bridge fdb append to 00:00:00:00:00:00 dst <peer-node-ip> dev vxlan100
ip link set vxlan100 master vrf-tenant-a
ip addr add 10.10.200.1/24 dev vxlan100
ip link set vxlan100 up

## 5. BGP Configuration

Configure FRR for BGP routing per tenant:
router bgp 65001
    bgp router-id 10.10.100.1
    neighbor 10.10.100.2 remote-as 65010
    address-family ipv4 unicast
    network 10.10.100.0/30
    exit-address-family

Bind BGP to VRF using vtysh:

vtysh -c "conf t" -c "router bgp 65001 vrf tenant-a"

## 6. Traffic Filtering

Use nftables to apply per-VRF firewall rules:
nft add table inet tenant_a
nft add chain inet tenant_a filter { type filter hook forward priority 0; }
nft add rule inet tenant_a filter ip saddr 10.10.100.0/30 counter accept

## 7. Outbound NAT Configuration

Assign unique public IPs per tenant for outbound NAT:
nft add rule inet tenant_a filter ip saddr 10.10.100.0/30 snat to 203.0.113.100

## 8. Loopback and DDNS Setup

Configure loopback IPs and DDNS for tunnel endpoints:
ip addr add 203.0.113.1/32 dev lo
Configure ddclient to update DNS records:
protocol=dyndns2
    use=web
    server=dyndns.example.com
    login=yourlogin
    password=yourpassword
    203.0.113.1