# Request for Proposal (RFP): A self-service consumable Third-Party Landing Zone (3PLZ) solution scalable to 1000 tenants for CVS Health

Friday, October 31, 2025

## 1. Executive Summary

Communication with our business partners is foundational to our success. Supporting this communication comes with some risk, this RFP in partnership with our vendors, looks to manage this risk in a transformative approach to delivering a connectivity solution to deliver a Third-Party Landing Zone (3PLZ) that enables secure, isolated, and auditable access between external vendors and enterprise applications.

This architecture embraces the best elements of network security best practice: least privilege, segmentation, intent, and identity. Delivering a business-to-business connectivity solution built on a high-availability, fault tolerant VPN model to support secure communications exchange. Terminating in a no-access tenant (Virtual Landing Zone – VLZ), in which application teams can provide pattern based, per-service connectivity through application-specific proxies governed by connectivity service profiles.

Considering size and scope of the current 3PLZ implementation one critical delivery aspect is requirement that the solution be largely automated and consumable as a self-service solution that can be directly consumed by application development teams, extending the DevOps philosophy of “build, run, own” into the delivery and consumption of network security. Embracing a self-service delivery approach, empowers the application teams that “own” the 3P connectivity, to migrate existing connectivity themselves, while ensuring that the best security practices are maintained through a delivery framework that can also deliver enhanced capabilities and controls as needed going forward.

## 2. Business Connectivity Objectives & Security Outcome Goals

The enterprise requires a repeatable, compliant, and automated approach for third-party access to SFTP, VDI, internal web apps, and external SaaS. This solution must maintain least privilege, auditable access while reducing operational overhead through automation and self-service onboarding. Key drivers include compliance with SOC 2 Type II, ISO 27001, and the NIST Cybersecurity Framework. Currently the enterprise supports the following 3PLZ application delivery profiles (going forward these will be referred to a services). Responses need to be able to be able to support the delivery of the following use cases in a manner that aligns with the relevant security standards.

### 2a. Use Case Scenarios and Security Best-Practice Alignment

### Secure File Transfer (SFTP/Sterling)

  - Use Case: Vendors exchange files via SFTP or IBM Sterling with PKI-secured sessions.

  - Relevant Standards: NIST SP 800-171 3.13, CIS Control 14, FIPS-compliant encryption.

### Collaboration Platform Access: (M365, eShare, Slack)

  - Use Case: External users access collaboration tools through proxied, audited channels.

  - Relevant Standards: ISO 27001 A.13.2.1, NIST AC-2 Account Management.

### Virtual Desktop Infrastructure (VDI)

  - Use Case: Third-party workers use Citrix/Microsoft VDI via per-tenant mTLS proxies.

  - Relevant Standards: NIST 800-207 ZTNA, ISO 27001 A.9.2, CIS Control 5.

- Web Application and API Integration for internal and external hosted Applications

  - Use Case: External apps integrate securely with enterprise APIs via mTLS and JWT.

  - Relevant Standards: NIST SP 800-95, OWASP API Top 10, CIS Control 16.

## 3. Architecture Overview

The challenge of 3PLZ (3rd Party Landing Zone) is not technical, as much as it is process. Delivering a solution that will scale and be manageable requires an investment in the operational and consumption aspects along with infrastructure. The consumers of the platform (i.e., Application Developers, managers of 3rd party staff/auditors/vendors and B2B relationships) need to have a solution that will deliver a positive user experience. Simply delivering a technical connectivity solution is not enough. Consider these user experience aspirations in your RFP response:

- The end-user does not require any technical understanding of the solution.

- The solution provides a set of defined capabilities. However, these capabilities can be expanded based on user demand.

- The solution can be consumed utilizing the Enterprises existing IAM capabilities and can support user to machine and machine to machine solution delivery.

- The solution provides the technical configuration for the entire B2B secure transport (i.e., The UI generates the IPSec Tunnel configuration required for the vendor or partner to establish connectivity based on the collection of a standard set of data points, that can be easily collected).

- The user is not responsible for the security compliance of the solution. Once the connectivity is in place, and application connections have been implemented, updates and enhancements to the different components of the service maintained in place with little to no service disruption greater than 5 minuets.

- The solution should be capable of utilizing distinct security credentials and credential types for each of the application use cases, as needed.

### 3a. B2B Connectivity Requirements

The Enterprise currently has a large exiting 3PLZ footprint, these connections will need to be migrated to the new platform. The current solution will require the implementation of a new IPSec Tunnel path (The partner side of this connection will be referred to as the RP and the enterprise side the LP). To ensure future portability, the LP side of the IPSec tunnel needs to be location/device agnostic, the public IP IPSec tunnel anchor addressing needs to be unique for each 3PLZ connection. The tunnel connection also needs to be portable; the goal is to provide a High Availability / Fault Tolerant connection. A partner can implement a two path, active/passive connectivity solution to the Enterprise (with path selection is based on peer remote peer path availability and endpoint route injection via private ASN BGP peering). On the enterprise side, the tunnel anchor IP needs to be portable. Movable between different IPSec gateways and available across multiple IP gateways (this capability can be delivered via floating IP Loopbacks for tunnel anchors or via IP Anycast). By adopting a connection solution strategy with connectivity portability as the primary goal, the opportunity cost required to move legacy connections is a one-time investment as opposed to an unplanned and unwelcomed periodic hidden cost.

#### 3a-1. Network Connectivity Functionality (Secure Transport Layer)

Communications between the enterprise and its business partners and vendors must be over a secure communications channel.  Each 3PLZ B2B connection will be supported over a IPSec/ISAKMPv2 Virtual Tunnel Interface supporting AES256 with GCM and a 96-bit Nonce.

The solution will provide the remote VPN device configuration for Cisco IOS XE, Arista EOS, Palo Alto Networks, Fortinet, and StrongSwan (Linux). This configuration will be available as part of the 3PLZ policy and deliverable via secure email, Web UI and API.

The VPN configuration includes:

- Tunnel Interface parameters

- ISAKMP & IPSec policies for negotiation, key exchange, encryption, and authentication

- PKI certificates for identity validation

- BGP routing configuration for dynamic route advertisement and reception

- Vendor Termination Equipment Model (Used for testing)

- Vendor Termination Equipment OS Version (Used for testing)

- Any additional supporting scripts if needed, configurations are pre-tested before distributed.

### 3b. B2B Traffic Isolation and 3PLZ tenants

There are technical hurdles that need to be addressed for any 3PLZ connection to function, which is commonly implemented as direct peer-to-peer IP connection.  This approach and the challenges that come with it, leave the Enterprise with risk and support management issues. That historically may have been acceptable but are no longer in today’s business security climate. Challenges supporting IP Routing due to overlapping IP address space, split horizon DNS and application traffic routing (today most Enterprises have a combination of on-prem and cloud application but only utilize a single DNS domain) are the most common. Meeting these challenges often requires bespoke fixes that are not sustainable and then turn into business risks. The critical risk that needs to be addressed is the problem of having a direct IP communications path between separate enterprises leaving both exposed in the event ether experience a security compromise.

Taking a page from the modern application delivery playbook, adopting per-partner, “tenant” delivery model for supporting 3PLZ connectivity. Enables the business to adopt ZTNA grounded approach to deliver secure, scalable and (most importantly) extensible application connectivity. What is a tenant? Each 3PLZ, B2B IPSec VPN connection has its own tenant. The tenant represents a logically isolated environment that defines who, what, and how communication to internal or external applications occurs. It combines three key components: Identity and Authorization, Network Connectivity, and Application Service Profiles. The following describes capability’s, operational and functional attributes that are expected to be part of purposed solutions.

#### 3b-1. Management and Configuration of VPN and Tenant

A Web UI is provided support the configuration and implementation of:

- IPSec Tunnels

- Definition 3PLZ B2B Tenants

- Definition of Tenant Services

- Definition of tenant Service-Profiles

- Administration of tenant attributes and core services

  - Tenant IP Blocks

  - Tenant IPSec Tunnel Addressing

  - Tenant IPSec Tunnel Anchor Addressing

  - Tenant Enterprise Service IP addressing

  - Tenant Internet Service IP addressing

  - Tenant BGP Service

  - Tenant DNS Service

  - Service Definition Security Policies

  - Service Definition IAM Policies

All the UI capabilities are also available via web api.

#### 3b-2. Identity & Authorization Capabilities

Web UI / api supports three role models Consumers, Administrators, Owner

There are three RBAC models read, read-write, and owner

The Owner RBAC role is limited to creation and deletion of tenants (users are added to this group) the group is owned by the application using the application ID as the contact

When a tenant is created, the identity context (owner ID, user ID, business/vendor ID, and application ID) is bound to that tenant.
mTLS certificates are used to provide Tenant/Service Profile identities to enterprise applications (where supported)

#### 3b-3. Service Definitions (Application Access Control Plane)

Every tenant can support the following 3PLZ application connectivity which is provided using application proxy services that are optimized to support the different application use cases. These service definitions are used to implement a given tenants service-profile (a logical policy that defines an application that is available to the users that have access to the tenant (i.e., business partner or vendor). The initial services definitions that need to be delivered are:

• SFTP / SSH — secure file transfer or command-line access
• VDI — virtual desktop access
• Web Application — HTTP/S front-end access
• Web API — REST or SOAP-based backend interfaces
• SaaS Services — Slack, O365, SharePoint, IBM Sterling File Gateway

There are two version of each service Enterprise and Internet. The distinction indicating the IP routing path that the service proxy traffic will follow.

3b-4. Service Profile Definitions (Tenant Application Access Policy)

A service profile needs to be defined for each 3PLZ application that can be accessed by the users with IP connectivity to the tenant. By default, no network level access control is enforced to limit application access. The capability however, to enforce such controls are required. Operationally, the service-policy is defined as follows:

Proxy rules and translation policies:

- A Service-Definition is selected (Enterprise or Internet),

- The enterprise FQDN for the resource is defined

- If additional controls are required, they are presented as options, relative to the Service Definition

Security group or firewall associations are applied based on the Service Definition selected

Routing / NAT configuration:

- 3PLZ Application Proxy IP addressing are allocated from the tenant pool.

- A DNS rewriting (and HTTP header if appropriate) rule is created based on the service-profile definition FQDN DNS name.

Application session monitoring parameters:

- IP level application proxy access logs are captured for each transaction

- X-Forwarded-For headers can also be used to capture origin IP addressing for security logging.

## 4. Core Component Minimum Capability Requirements and Security Best Practices Control Mappings

The following tables define the core components, technical need and security compliance justification that expect to be met as part of any minimum viable product solution. Commercial and open-source product references are included in some of the component definitions these are provided to provide an industry capability benchmark. RFP responses do not need to include or utilize any of the products referenced, only the functions or capabilities.

| Component | Technical Purpose | Security & Compliance Justification |
| --- | --- | --- |
| IPsec IKEv2 Tunnel | Creates authenticated encrypted tunnels between gateways. | NIST SP 800-77, RFC 7296, FIPS 140-3 compliant encryption. |
| No-Access Tenant (VLZ) | Terminates tunnels without exposing internal networks. | NIST 800-207 Zero Trust principle; CIS Control 13 Segmentation. |
| Application Proxy (Envoy/HA Proxy) | Per-service gateway with mTLS enforcement. | NIST AC-4 and ISO 27001 A.13.1.3 Network Segregation. |
| VDI Proxy (v2rayA, Shells) | Per tenant support for 3rd party VDI access. | NIST AC-4 and ISO 27001 A.13.1.3 Network Segregation. |
| SSH/SFTP Proxy (OpenSSH. HA Proxy) | Per tenant support for 3rd party SSH and SFTP access. | NIST AC-4 and ISO 27001 A.13.1.3 Network Segregation. |
| BGP Per-Service Routing | Advertises only service-specific prefixes. | CIS Control 6 Least Privilege Access. ISO 27001 A.13.1.3 |
| Split-Horizon DNS (Core DNS, BIND) | Resolves internal vs public paths via policy. | NIST SI-4 Monitoring, RFC 8499 DNS Security. |
| PKI / mTLS Identity | Authenticates all connections using certificates. | NIST IA-2, ISO 27001 A.9.4 Access Control. |
| Automation & API for Tenant and IPSec Tunnel provisioning | Enables Terraform/IaC provisioning. | NIST SP 800-218 DevSecOps and CIS Control 4. |
| Observability & Logging | Exports per-tenant logs and metrics to SIEM. | NIST AU-12, ISO 27001 A.12.4 Logging and Monitoring. |
| Per Tenant DNS Services and re-writing, via upstream declarative forwarding | Provide DNS resolution IP addressing abstraction while using common DNS FQDN references. | CIS Control 6 Least Privilege Access. |
| NIST SP 800-57 | PKI Lifecycle Automation | Certificate Repository & Rotation Policies |

## 5.0 Contact for Questions

## Any question or clarifications regarding the technical or user experience architecture should be sent to:

Michael J Martin

Michael.Martin4@cvshealth.com

203 300 6446

## 6. Appendix

### 5.1 Examples of IPSec VTI Configuration:

Cisco IOS XE:

! Phase 1 - IKEv2 Proposal, Policy, and Keyring

!

crypto ikev2 proposal IKEV2-PROPOSAL

encryption aes-gcm 256

group 14

! The integrity algorithm is omitted because GCM provides built-in integrity.

exit

crypto ikev2 policy IKEV2-POLICY

proposal IKEV2-PROPOSAL

exit

crypto ikev2 keyring IKEV2-KEYRING

peer PEER1

address <peer-ip-address>

pre-shared-key local <your-pre-shared-key>

pre-shared-key remote <remote-pre-shared-key>

exit

crypto ikev2 profile IKEV2-PROFILE

match identity remote address <peer-ip-address> 255.255.255.255

identity local address

keyring local IKEV2-KEYRING

authentication local pre-share

authentication remote pre-share

exit

!

! Phase 2 - IPSec Transform-Set and Profile

!

crypto ipsec transform-set IPSEC-TS esp-aes-256 esp-gcm 256

mode tunnel

! The "esp-gcm 256" implicitly defines the 96-bit nonce and GCM authentication.

exit

crypto ipsec profile IPSEC-PROFILE

set transform-set IPSEC-TS

set ikev2-profile IKEV2-PROFILE

set pfs group 14

exit

!

! VTI Interface Configuration

!

interface Tunnel1

ip address <local-tunnel-ip> <tunnel-subnet-mask>

tunnel mode ipsec ipv4

tunnel source <local-public-ip>

tunnel destination <peer-ip-address>

tunnel protection ipsec profile IPSEC-PROFILE

no shutdown

exit

!

! Routing (Example: Static routes for peer's protected subnets)

!

ip route <remote-protected-network> <remote-wildcard-mask> Tunnel1

!

end

Arista EOS:

! --------------------------------------------------

! 1. Define IKEv2 Proposal (Phase 1)

! --------------------------------------------------

! GCM provides combined encryption and integrity.

crypto ikev2 proposal IKEV2_PROPOSAL

encryption aes-gcm-256

group 14

! The integrity algorithm is omitted because GCM handles it internally.

! --------------------------------------------------

! 2. Define IKEv2 Policy

! --------------------------------------------------

! Apply the proposal to a policy.

crypto ikev2 policy IKEV2_POLICY

proposal IKEV2_PROPOSAL

! --------------------------------------------------

! 3. Define IKEv2 Keyring

! --------------------------------------------------

! Configure pre-shared key authentication.

crypto ikev2 keyring IKEV2_KEYRING

peer PEER1

address <Peer_Public_IP>/32

pre-shared-key local <Pre-Shared_Key>

pre-shared-key remote <Pre-Shared_Key>

! --------------------------------------------------

! 4. Define IKEv2 Profile

! --------------------------------------------------

! Link the keyring and set peer matching criteria.

crypto ikev2 profile IKEV2_PROFILE

match identity remote address <Peer_Public_IP>/32

identity local address

keyring local IKEV2_KEYRING

authentication local pre-share

authentication remote pre-share

! --------------------------------------------------

! 5. Define IPSec Transform Set (Phase 2)

! --------------------------------------------------

! This defines the data plane encryption (AES256-GCM).

crypto ipsec transform-set IPSEC_TRANSFORM_SET

protocol esp encryption aes-gcm-256

mode tunnel

! NOTE: Arista EOS automatically handles the GCM IV/nonce requirements using the standard 96-bit length when 'aes-gcm-256' is specified.

! --------------------------------------------------

! 6. Define IPSec Profile

! --------------------------------------------------

! Link the transform set and the IKEv2 profile.

crypto ipsec profile IPSEC_PROFILE

set transform-set IPSEC_TRANSFORM_SET

set ikev2-profile IKEV2_PROFILE

set pfs group 14

! --------------------------------------------------

! 7. Configure the Tunnel Interface

! --------------------------------------------------

! Create the VTI interface and apply the configuration.

interface Tunnel1

ip address <Local_Tunnel_IP>

tunnel source <Local_Public_IP>

tunnel destination <Peer_Public_IP>

tunnel mode ipsec

tunnel protection ipsec profile IPSEC_PROFILE

no shutdown

! --------------------------------------------------

! 8. Add Static Routes (or dynamic routing)

! --------------------------------------------------

! Route traffic for the remote internal network (e.g., 192.168.1.0/24) via the tunnel IP.

ip route 192.168.1.0/24 <Remote_Tunnel_IP>

StrongSwan

1. /etc/ipsec.conf This file defines the connection parameters. Replace the bracketed variables with your actual IPs and interface names.ini

# /etc/ipsec.conf - strongSwan IPsec configuration file

config setup

# strictcrlpolicy=yes

uniqueids=no

charondebug="ike 2, knl 2, cfg 2"

conn VTI-TUNNEL-GCM

# Basic setup

keyexchange=ikev2

type=tunnel

auto=start

# Prevents strongSwan from managing routes and policy based on subnets

# We will manually manage the VTI interface and routes

leftupdown=/etc/ipsec.d/vti-up.sh

# Dead Peer Detection (DPD) settings

dpdaction=restart

dpddelay=10s

dpdtimeout=30s

# Local side (left)

left=<Local_Public_IP>

# Use 0.0.0.0/0 for route-based VPN traffic selectors

leftsubnet=0.0.0.0/0

# Remote side (right)

right=<Peer_Public_IP>

rightsubnet=0.0.0.0/0

# IKEv2 Phase 1 proposals: Encryption (AES-GCM-256), DH Group 14

# GCM implies authenticity/integrity

ike=aes256gcm16-modp2048!

# IPSec Phase 2 proposals: Encryption (AES-GCM-256)

# 'aes256gcm16' uses AES-GCM-256 with a 16-byte (128-bit) Integrity Check Value (ICV)

# Note: strongSwan handles the 96-bit nonce length automatically with this setting

esp=aes256gcm16!

2. IPSec Secrets. This file stores your pre-shared key.

# /etc/ipsec.secrets - strongSwan secrets file

# <Local_Public_IP> <Peer_Public_IP> : PSK "<Pre-Shared_Key>"

<Local_Public_IP> <Peer_Public_IP> : PSK "<Pre-Shared_Key>"

3. VTI Interface Script You need a script to manage the VTI interface dynamically when the tunnel comes up or goes down. Create /etc/ipsec.d/vti-up.sh and make it executable:

#!/bin/bash

# /etc/ipsec.d/vti-up.sh

VTI_IF="vti0" # Can be any name, but make sure it's unique if using multiple tunnels

LOCAL_TUNNEL_IP="<Local_Tunnel_IP>" # e.g., 10.0.0.1/30

REMOTE_TUNNEL_IP="<Remote_Tunnel_IP>" # e.g., 10.0.0.2

case "${1}" in

up-client)

# Add the VTI interface and configure IPs

ip tunnel add "${VTI_IF}" mode vti local "${PLUTO_ME}" remote "${PLUTO_PEER}" okey "${PLUTO_MARK_OUT%%/*}" ikey "${PLUTO_MARK_IN%%/*}"

ip link set "${VTI_IF}" up

ip addr add "${LOCAL_TUNNEL_IP}" dev "${VTI_IF}"

# Add a route for the remote network via the tunnel interface's remote IP

ip route add <Remote_Protected_Network> via "${REMOTE_TUNNEL_IP}" dev "${VTI_IF}"

;;

down-client)

# Delete the VTI interface when the tunnel goes down

ip tunnel del "${VTI_IF}"

;;

esac

The platform must enforce Zero-Trust segmentation—no direct layer-3 access between remote and corporate networks—and align with NIST SP 800-207, ISO/IEC 27001:2022 A.5/A.8/A.12, and CIS Controls v8 #3 (Access Control).

Examples of configs are in the appendix section.

The tenant in relationship to the IPSec Tunnel, represents an IP network segment that supports the service-profile connections IP addresses